MECHANICAL DESCRIPTION OF BOARD

MAX2640 EVALUATION BOARD REV-A

This is a 4 layer 50 OHM controlled impedence board, the Dielectric material is FR4, 1 oz copper

with a dielectric constant of ~ 4.5.  The Dielectric thickness between top layer and internal Gnd

plane (layer 2) must be 24 mil.

| Material: | Epoxy Glass FR4 |
| --- | --- |
| Size: | 1.40”X2.475” |
| Thickness: | 0.062” +/-0.003” |
| Layers: | 4 |
| Solder Mask: | LPI SMOBC |
| Legends: | White (all ink must be clipped from expose metal) |
| Copper Clad: | 1 OZ |

VENDOR LOGO & DATE CODE:	Allow only in Ink on bottom side only.

PLATING:

| Thru Holes: | 0.001” min |
| --- | --- |
| Tin-Lead: | 0.0002” min |

SURFACE MOUNT PADS:			25

TOLERENCES:

| Board Dimensions | +/- 0.010” |
| --- | --- |
| Plated-Thru Holes: | +/- 0.003” |
| Pattern-to Pattern | +/- 0.005” |
| Solder Mask to Pattern | +/- 0.005” |
| Legend to Legend | +/- 0.007” |

Drill Schedule is located on the Drill Plot.

Erik C. Cheung

Maxim Integrated Products

120 San Gabriel Dr.

Sunnyvale CA 94086

(408)530-6214

Fax (408)732-8159

10/30/98