MECHANICAL DESCRIPTION OF BOARD

MAX7057 EVALUATION KIT+ REV-A

| Material: | RoHS Compliant FR-408 or similar laminate material compatible with Lead-Free soldering processes |
| --- | --- |
| Size: | 6.50”X 3.20” |
| Thickness: | 0.062” |
| Layers: | 2 |
| Trace/Space: | 8 mils |
| Solder Mask: | GREEN LPI SMOBC |
| Legends: | White. Double-sided (clipped all legends from exposed metal) |
| Copper Clad Finish: | 1 oz minimum for all layers |

VENDOR LOGO & DATE CODE:	Allowed in Ink on bottom side only.

| PLATING | Must be Lead free and RoHS compliant |
| --- | --- |
| Finish: | Vendor should use the most economical Lead Free and RoHS compliant process available or as specified in Purchase Order. Approved Finish: HASL Lead-Free solder Immersion Tin Immersion Gold |
| Thru Holes: | 0.001” min |
| Quality: | Manufactured in accordance with IPC-A-600 |

SURFACE MOUNT PADS:			464

TOLERANCES:

| Board Dimensions | +/- 0.010" |
| --- | --- |
| Plated-Thru Holes: | +/- 0.003" |
| Pattern-to Pattern | +/- 0.005" |
| Solder Mask to Pattern | +/- 0.005" |
| Legend to Legend | +/- 0.007" |

Drill Schedule is located on the Drill Plot.