test

Product qualification

report

TITLE:

Test Site Transfer of Products

From ADGT to STATSChipPAC, China

REPORT NUMBER:

REVISION: B

DATE:

8/5/2010

SUMMARY

STATSChipPAC, China is an ADI qualified test site running various ADI products. With the increasing ADI volume, ADI products have been identified for transfer for additional test capacity.

There is no change to the form, fit, function, quality or reliability of the transferred parts.

This report documents the successful   completion of the product test transfer requirements for the release of  various products with ADM1485 as product qualifier.

Test product qualification was performed according to Analog Devices Specification

(ADI0012 / TST000137)

TEST AND PRODUCT INFORMATION

Device: 					ADM1485

Package: 				SOIC
Leads (if applicable):

Part names:

TEST_PART			FG_PART

ADM1485ARU-T0JCO		ADM1485AR

ADM1485AR3-T0JCO		ADM1485AR-REEL

ADM1485AR7-T0JCO		ADM1485AR-REEL7

ADM1485ARU-T0JCOZ		ADM1485ARZ

ADM1485AR3-T0JCOZ		ADM1485ARZ-REEL

ADM1485AR7-T0JCOZ		ADM1485ARZ-REEL7

ADM1485JRU-T0JCO		ADM1485JR

ADM1485JR3-T0JCO		ADM1485JR-REEL

ADM1485JR7-T0JCO		ADM1485JR-REEL7

ADM1485JRU-T0JCOZ		ADM1485JRZ

ADM1485JR3-T0JCOZ		ADM1485JRZ-REEL

ADM1485JR7-T0JCOZ		ADM1485JRZ-REEL7

Testers: 				Teradyne MicroFlex MS

Handler: 				Mulititest

DUT board number of test Sites: 	4

Description and Test Results ( Taken from the New Proposed Product Transfer Correlation Qual Criteria)

Table 1 provides a description of the qualification tests conducted and corresponding test results for ADM1485. All the units have undergone electrical tests on both the sending and receiving sites on the same test platform.  Any device that did not meet the electrical qualification requirements without further analysis and data to prove passing the qualification would be considered failed.

Table 1. Test Product Transfer Qual Criteria

| Generic | Package | Sending Site | Receiving Site | GB Comparison SCC GB < ADGT GB or SCC GB < ADGT GB | Passing Unit Correlation, 10 units | Reject Unit Correlation, 5 units |
| --- | --- | --- | --- | --- | --- | --- |
| ADM1485 | SOIC | ADGT | STATSChipPAC, China | PASSED | PASSED | PASSED |

Table 2  shows results of the qualification lot run . The qualification lot has undergone electrical test on both the sending site and receiving site on the same test platform. Any deviation on the lot qualification run criteria without further analysis and data to prove a passing qualification would be considered a failed qualification lot run.

Table 2. Test Product Transfer Qualification Lot Run

| GENERIC | Package | Lot Size | Sending Site | Receiving Site | Comparison of yields between two sites within 2% |
| --- | --- | --- | --- | --- | --- |
| ADM1485 | SOIC | 250 units | ADGT | STATSChipPAC, China | PASSED |

Approvals

TRB.

Test Product Engineer :

Supporting Documents

Customer Notification:

Technical Review Board: TRB#4137

Additional Information

Homepage:

Datasheet:

Customer Service: