# CN0343 PCB illustration

- Board to be accord with ROHS(Restriction of Hazardous Substances).

- PCB Material: FR-4.

- Board type: 2 copper layers, from top to down: topover, topsolder, top, bottom, bottomsolder, bottomover.

- Default tolerance: take 10% or 0.1mm whichever is minor

- Board Thickness: 1.6mm.

- Copper layers thickness: 1oz after processing and plating, about 0.5oz substrates, 0.5oz plated.

- Slot holes: this board maybe contain the slot holes, it will be define in the drill drawing files.

- Holes Planting: according to the drill file, approximate 0.5oz, average 25um. No special treat, same as layers plant.

- Surface finishes: Lead free HASL(hot air solder leveling).

- Solder mask: green, on both sides per gerber files.

- Vias tenting: via solder mask tenting according to the gerber files.

- Silkscreen: white, on both sides per gerber files.

- Additional: Some copper maybe over the board edge, just cut away.

- If the unit board is not large enough to production. For cut the cost, multiple pieces joint is allowed, but it need to cut each apart at delivery.

- Any questions please email to: Robert.Lee@Analog.com