Parts: MAX17502, MAX3390, MAX7219

Meta Words: MAXREFDES99, Display driver, LED, 8x8 Matrix, Shield, MBED, Arduino

Meta Description: MAXREFDES99# is an easy to use and flexible development board to support rapid prototyping for the MAX7219 LED display driver using ARM microcontroller platforms supported by the online development environment ARM® mbed™ and Arduino platforms.

Abstract: MAXREFDES99# is an Arduino form factor compatible shield for the rapid development of LED displays using the MAX7219 LED display driver.

MAXREFDES99#: MAX7219 Display Driver Shield

Overview

Maxim’s MAXREFDES99# is an Arduino form factor shield which drives a 16 x 16 LED array for signage applications.  This design is ideal for anyone developing display application.  The board features Maxim’s MAX7219, a legendary LED driver which simplifies the process of controlling multiple LEDs in matrix or numeric display form.  In MAXREFDES99#, four MAX7219 ICs are daisy chained to drive a full 256 LED array which may display any international alpha numeric symbol.  The board arrives complete, with four 8x8 LED displays.

Code for the MAXREFDES99# is available on both Arduino and mbed.org platforms for quick prototyping with multiple platform boards.  System demonstrations include the display of ASCII characters ‘SPACE’ through ‘~’ and writing/scrolling string messages.  Developers are encouraged to build upon and enhance the code base.

Features

- Flexible 16x16 dot matrix display

- mbed.org library

- Arduino library

- Arduino® shield form factor

Applications

- Displays

- Consumers

- Robotics

Competitive Advantages

- Portable

- Compact

- Flexible

- Cost Effective

Introduction

LEDs, both low cost and available in many configurations, continue to dramatically transform signage and lighting.  The MAX7219 has long been the developer’s best friend in LED driving, offering both simplicity and flexibility.  Most systems, however, drive 8x8 arrays, limiting the amount of communication, specifically character size, and not utilizing the daisy chain feature of the MAX7219.  16x16 displays, in contrast, provide the capability to display most international alpha-numeric characters.

Enter the MAXREFDES99#, which drives a 16x16 display, making it a powerful tool for prototyping and developing applications, which requires displays.

The MAXREFDES99# is mbed and Arduino compatible, allowing fast development for both platforms. The shield contains four daisy-chained MAX7219s that drive four 8x8 red led dot matrix displays installed on the top arranged to make a 16x16 matrix display.

Detailed Description of Hardware

Please see Figure 1 for a block diagram of MAXREFDES99#.

MAXREFDES99# can be powered from a wall-wart which provides a minimum of 7W of power and an output voltage in the range of 7.5-12VDC.  7W is enough power to power the display, plus the microcontroller platform being used through the Vin pin of the target platform.  The reference design can be optionally powered from a USB port while developing application code, however, the user must take care not to exceed the power capabilities of the 5V rail supplied by their platform.  The recommended method of powering MAXREFDES99# is via a wall-wart connected to J1.

MAXREFDES99# uses the MAX3390E level translator to provide a 3.3V logic level  compatible interface to the four MAX7219 display drivers when using the shield with 3.3V platforms.  The serial interface between the platform and the shield uses D10, D11, and D13 for LOAD, DIN, and CLK respectively.  The four MAX7219 display drivers are daisy-chained connected with D11 connected to U3 and DOUT of U3 connected to DIN of U4 and so on.  DOUT of U6 is left unconnected.

Figure 1: MAXREFDES99# block diagram

Detailed Description of Firmware

Libraries have been developed for both Arduino platforms and the online development environment mbed.org which supports microcontrollers from several manufacturers of ARM Cortex-M microcontrollers.  The source code for these libraries and their corresponding ‘Hello World’ demo programs can be found in the design files, or at the following links.

- Arduino

  - GitHub Link (Update with GitHub link when available)

- Mbed

  - Library

  - Demo Program

Figure 2 provides a high-level flow chart of the demo program for both platforms.

Figure 2:  Demo program flowchart

Quick Start

Required equipment:

- Windows® PC with a USB port

- MAXREFDES99#

- Compatible Arduino® form factor mbed™ platform

- 7W min Wall-Wart with 7.5 to 12VDC output

Procedure:

The reference design is fully assembled and tested.  Follow the steps below to verify board operation.

- Connect MAXREFDES99# to your platform.

- Connect wall-wart to J1 of MAXREFDES99#

- Connect USB cable from the PC to your platform.

- Follow the appropriate instructions below for your platform.

  - Arduino

    - Download the library from the GitHub repository (update with link when available)

    - Follow the instructions here for installing third party libraries.

    - Open your Arduino IDE, version 1.6.7 or later.

    - Navigate to File/Examples/MAX7219/MAXREFDES99_example

  - Mbed

    - If you have never used mbed, please setup an account and add your platform to your online compiler per the instructions on your platform’s product page.

    - If you are using Windows and have never used serial communications between your PC and your mbed platform, please follow the instructions here.

    - Navigate your browser to this link.

    - Click the ‘Import this program’ button and import the demo as a ‘program’

- Compile and download the resulting binary to your platform.

- Open Hyperterminal, or your favorite terminal emulator, find the appropriate COM port for your platform, and configure the connection for 9600bps, 8-N-1 with no flow control.

- Press the reset button on your platform and exercise the demo.

Lab Measurements

All Design Files

Buy Reference Design

MAXREFDES99#

Trademarks

ARM and MBED are registered trademarks of ARM Holdings.

Arduino is a registered trademark of Arduino LLC.

Reference