Figure 1: Program Listing

/***************************************************************************/

/* DEMO1302.C                                                              */

/***************************************************************************/

/* This program is for example only and is not supported by Dallas Maxim   */

#include	<stdio.h>

#include	<DS5000.h>		/* Register declarations for DS5000     */

sbit CE   = P0^0;

sbit SCLK = P0^1;

sbit IO   = P0^2;

/***************************************************************************/

/* Prototypes                                                              */

/***************************************************************************/

uchar	rbyte_3w();

void	reset_3w();

void	wbyte_3w(uchar);

void	writebyte();

void	write_clk_regs();

void	read_clk_regs();

void	burstramrd();

void	burstramwr();

/***************************************************************************/

void reset_3w()	/* ----- reset and enable the 3-wire interface ------ */

{

SCLK = 0;

CE = 0;

CE = 1;

}

void wbyte_3w(uchar W_Byte)	/* ------ write one byte to the device ------- */

{

uchar i;

for(i = 0; i < 8; ++i)

{

IO = 0;

if(W_Byte & 0x01)

{

IO = 1;	/* set port pin high to read data */

}

SCLK = 0;

SCLK = 1;

W_Byte >>= 1;

}

}

uchar	rbyte_3w()	/* ------- read one byte from the device -------- */

{

uchar i;

uchar R_Byte;

uchar TmpByte;

R_Byte = 0x00;

IO = 1;

for(i = 0; i < 8; i++)

{

SCLK = 1;

SCLK = 0;

TmpByte = (uchar)IO;

TmpByte <<= 7;

R_Byte >>= 1;

R_Byte |= TmpByte;

}

return R_Byte;

}

void	writebyte()	/* --- write one byte using values entered by user --- */

{

uchar ClkAdd;

uchar ClkData;

/* Get Clock Address & Data */

printf("\nEnter register write address (hex): ");

scanf("%bx", &ClkAdd);

printf("Enter data (hex): ");

scanf("%bx", &ClkData);

reset_3w();

wbyte_3w(ClkAdd);

wbyte_3w(ClkData);

reset_3w();

}

void read_clk_regs()	/* ---- loop read & display clock registers ---- */

{

uchar prev_sec = 99, sec, min, hrs, dte, mon, day, yr;

printf("\nYr/Mn/Dt Dy Hr/Mn/Sec");

do	/* Read & Display Clock Registers */

{

reset_3w();

wbyte_3w(0xBF);	/* clock burst */

sec = rbyte_3w();

min = rbyte_3w();

hrs = rbyte_3w();

dte = rbyte_3w();

mon = rbyte_3w();

day = rbyte_3w();

yr  = rbyte_3w();

reset_3w();

if(sec != prev_sec)	/* print once per second */

{

printf("\n%02bX/%02bX/%02bX %02bX", yr, dte, mon, day);

printf(" %02bX:%02bX:%02bX", hrs, min, sec);

prev_sec = sec;

}

}	while(!RI);

RI = 0;	/* clear read buffer */

}

void burstramrd()	/* ----------- read RAM in burst mode --------------- */

{

uchar i;

printf("\nDS1302 Ram");

reset_3w();

wbyte_3w(0xFF);	/* RAM burst read */

for (i = 0; i < 31; i++)

{

if(!(i % 8)) printf("\n");

printf("%2.bX ", rbyte_3w() );

}

reset_3w();

}

void burstramwr()	/* ---- write one value entire RAM in burst mode ---- */

{

uchar ramdata;

uchar	i;

printf("\nWrite Ram DATA (HEX):");		/* Get Ram Data */

scanf("%bx", &ramdata);

reset_3w();

wbyte_3w(0xfe);	/* RAM burst write */

for (i=0; i<31; ++i)

{

wbyte_3w(ramdata);

}

reset_3w();

}

void	write_clk_regs()	/* --- initialize time & date from user entries --- */

/* Note: NO error checking is done on the user entries! */

{

uchar	yr, mn, date, dy, hr, min, sec, day;

printf("\nEnter the year (0-99): ");

scanf("%bx", &yr);

printf("Enter the month (1-12): ");

scanf("%bx", &mn);

printf("Enter the date (1-31): ");

scanf("%bx", &date);

printf("Enter the day (1-7): ");

scanf("%bx", &dy);

printf("Enter the hour (1-24): ");

scanf("%bx", &hr);

hr = hr & 0x3f;	/* force clock to 24 hour mode */

printf("Enter the minute (0-59): ");

scanf("%bx", &min);

printf("Enter the second (0-59): ");

scanf("%bx", &sec);

reset_3w();

wbyte_3w(0x8e);	/* control register */

wbyte_3w(0);		/* disable write protect */

reset_3w();

wbyte_3w(0x90);	/* trickle charger register */

wbyte_3w(0xab);	/* enable, 2 diodes, 8K resistor */

reset_3w();

wbyte_3w(0xbe);	/* clock burst write (eight registers) */

wbyte_3w(sec);

wbyte_3w(min);

wbyte_3w(hr);

wbyte_3w(date);

wbyte_3w(mn);

wbyte_3w(dy);

wbyte_3w(yr);

wbyte_3w(0);		/* must write control register in burst mode */

reset_3w();

}

main	(void)	/* ---------------------------------------------- */

{

uchar	M, M1;

while (1)

{

printf("\nDS1302 build: %s\n", __DATE__);

printf("CW Clock Write CR Clock Read\n");

printf("RW RAM Write   RR RAM Read\n");

printf("W  Write byte\n");

printf("Enter Menu Selection: ");

M = _getkey();

switch(M)

{

case 'C':

case 'c':

printf("\rEnter Clock Routine to run:C");

M1 = _getkey();

switch(M1)

{

case 'R':

case 'r':	read_clk_regs();	break;

case 'W':

case 'w':	write_clk_regs();	break;

}

break;

case 'R':

case 'r':

printf("\rEnter Ram Routine to run:R");

M1 = _getkey();

switch(M1)

{

case 'R':

case 'r':	burstramrd();	break;

case 'W':

case 'w':	burstramwr();	break;

}

break;

case 'W':

case 'w':	writebyte();	break;

}

}

}