/***************************************************************************/

/* DEMO1305.C - This file is provided to show an example of routines       */

/* for interfacing the DS2250 to the DS1305.                              */

/* These routines are provided for example only and are not supported by   */

/* by Dallas Semiconductor MAXIM.                                          */

/***************************************************************************/

#include <stdio.h>		/* Prototypes for I/O functions            */

#include <DS5000.h>		/* Register declarations for DS2250        */

/************************** Function Prototypes ****************************/

void	reset_3w(void);

void	WBYTE_3W(uchar W_Byte);

uchar	RBYTE_3W(void);

void	Initialize();

void	BurstReadTime();

void	BurstRamRead(void);

void	BurstRamWrite(void);

void	writebyte1305();

void	readbyte1305();

void	alarm_set();

void	alarm_rd();

/************************** Bit Declarations *******************************/

sbit	CE        = P0^0;	/* to CE pin on DS1305 */

sbit	SCLK      = P0^1;	/* to SCLK pin on DS1305 */

sbit	SDI_IO    = P0^2;	/* to SDI & SDO pins on DS1305 */

sbit	INT0b     = P3^2;	/* to INT0b pin on DS1305 */

/************************** Global Variables *******************************/

uchar CPOL = 0;

void reset_3w()

{

SCLK  = 0;

CE    = 0;

CE    = 1;

}

void WBYTE_3W(uchar W_Byte)	/* --- This routine writes one byte from the RTC --- */

{

uchar i;

for(i = 0; i < 8; ++i)

{

SDI_IO = 0;

if(W_Byte & 0x01)

{

SDI_IO = 1;

}

SCLK = 0;

SCLK = 1;

W_Byte >>= 1;

}

}

uchar RBYTE_3W()	/* ---- This routine reads one byte from the RTC ----- */

{

uchar i, R_Byte, TmpByte;

R_Byte = 0x00;

SDI_IO = 1;	/* set to read */

for(i = 0; i < 8; ++i)

{

SCLK = 1;

SCLK = 0;

TmpByte = (uchar)SDI_IO;

TmpByte <<= 7;

R_Byte >>= 1;

R_Byte |= TmpByte;

}

return R_Byte;

}

void readbyte1305()	/* ------------------------------------------------ */

{

uchar Add;

uchar Data;

/* Get Address & Data */

printf("\nEnter the Read Address (hex): ");

scanf("%bx", &Add);

reset_3w();

WBYTE_3W(Add);

printf("%2.bx", RBYTE_3W() );

reset_3w();

}

void writebyte1305()	/* ------------------------------------------------ */

{

uchar Add, Data;

/* Get Address & Data */

printf("\nEnter the Read Address (hex): ");

scanf("%bx", &Add);

printf("Enter data (hex): ");

scanf("%bx", &Data);

/* User enters read address. Must convert to write address. */

Add |= 0x80;

reset_3w();

WBYTE_3W(Add);

WBYTE_3W(Data);

reset_3w();

}

void Initialize()	/* ------------------------------------ */

{

/* Note: NO error checking is done on the user entries! */

uchar	yr, mn, dt, dy, hr, min, sec, day;

reset_3w();	/* Read contents of Write Protect Register */

WBYTE_3W(0x8F);

WBYTE_3W(0);	/* if write protect bit is set, this will only clear WP */

reset_3w();

WBYTE_3W(0x8F);

WBYTE_3W(0);	/* if WP was set, now /eosc will be set to a zero */

reset_3w();	/* if we were setting WP to  a 1, we could set all bits at once */

printf("\nEnter the year (0-99): ");

scanf("%bx", &yr);

printf("Enter the month (1-12): ");

scanf("%bx", &mn);

printf("Enter the date (1-31): ");

scanf("%bx", &dt);

printf("Enter the day (1-7): ");

scanf("%bx", &dy);

printf("Enter the hour (1-24): ");

scanf("%bx", &hr);

hr = hr & 0x3f;	/* force clock to 24 hour mode */

printf("Enter the minute (0-59): ");

scanf("%bx", &min);

printf("Enter the second (0-59): ");

scanf("%bx", &sec);

reset_3w();

WBYTE_3W(0x80);	/* write address for the seconds register */

WBYTE_3W(sec);

WBYTE_3W(min);

WBYTE_3W(hr);

WBYTE_3W(dy);

WBYTE_3W(dt);

WBYTE_3W(mn);

WBYTE_3W(yr);

reset_3w();

}

void BurstReadTime()	/* ------------------------------------ */

{

uchar Sec, Min, Hrs, SavHrs, Day, Dte, Mon, Yr;

reset_3w();

WBYTE_3W(0x00);

Sec = RBYTE_3W();

Min = RBYTE_3W();

Hrs = RBYTE_3W();

Day = RBYTE_3W();

Dte = RBYTE_3W();

Mon = RBYTE_3W();

Yr  = RBYTE_3W();

reset_3w();

SavHrs = Hrs;

/* If 12hr mode clear bits 5 & 6 of hours register for display */

if (SavHrs & 0x40)

Hrs = Hrs & 0x1F;

printf("\r%02bX/%02bX/%02bX %1bX ", Yr, Mon, Dte, Day);

printf("%02bX:%02bX:%02bX", Hrs, Min, Sec);

/* If 12hr mode display  AM/PM */

if (SavHrs & 0x40)

{

if ((SavHrs & 0x60) == 0x60)

printf("PM");

else

printf(" AM");

}

}

void BurstRamRead()	/* ------------------------------------ */

{

uchar keypress;

uchar i;

uchar j;

reset_3w();

WBYTE_3W(0x20);

for (i = 0; i < 6; ++i)

{

printf("\n");

for (j = 0; j < 16; ++j)

{

printf("%2.bX ", RBYTE_3W() );

}

}

reset_3w();

}

void BurstRamWrite()	/* ---------- fill RAM with 0xAA ----------- */

{

uchar i, dat;

printf("\nEnter RAM data (hex): ");

scanf("%bx", &dat);

reset_3w();

WBYTE_3W(0xA0);

for (i = 0; i < 96; ++i)

{

WBYTE_3W(dat);

}

reset_3w();

}

void	alarm_set()	/* ------- set up alarm 0 for once-per-second ------ */

{

reset_3w();

WBYTE_3W(0x87);	/* write address for sec alarm 0 register */

WBYTE_3W(0x80);	/* set mask bit */

WBYTE_3W(0x80);

WBYTE_3W(0x80);

WBYTE_3W(0x80);

reset_3w();

reset_3w();

WBYTE_3W(0x8f);	/* write address for control register */

WBYTE_3W(0x05);	/* set INTCN, AIE0--assumes WP is already clear */

reset_3w();

}

void	alarm_rd()	/* ---- wait for int, read time * date, repeat --- */

{

do

{

while(INT0b);	/* wait until RSTb goes active */

BurstReadTime(); /* read time & date, this also clears alarm 0 */

reset_3w();

WBYTE_3W(0x07);	/* read address for alarm 0 sec register */

RBYTE_3W();		/* read alarm 0 sec reg to clear INT0 */

reset_3w();

}	while(!RI);

RI = 0;	/* clear buffer */

}

void	main()	/* --------------- main ----------------- */

{

uchar i, M, M1;

while (1)

{

printf("\nDS1305 build: %s\n", __DATE__);

printf("AR Alarm read AS Alarm set\n");

printf("CR Clock Read CI Clock Init\n");

printf("BR Byte Read  BW Byte Write\n");

printf("RR RAM Read   RW RAM Write\n");

printf("Enter Menu Selection: ");

M = _getkey();

switch(M)

{

case 'A':

case 'a':

printf("\rEnter alarm routine to run:A");

M1 = _getkey();

switch(M1)

{

case 'R':

case 'r':	alarm_rd();	break;

case 'S':

case 's':	alarm_set();	break;

}

break;

case 'B':

case 'b':

printf("\rbyte R)ead or W)rite? : ");

M1 = _getkey();

switch(M1)

{

case 'R':

case 'r':	readbyte1305();	break;

case 'W':

case 'w':	writebyte1305();	break;

}

break;

case 'C':

case 'c':

printf("\rEnter Clock Routine to run:C");

M1 = _getkey();

switch(M1)

{

case 'R':

case 'r':	BurstReadTime();	break;

case 'I':

case 'i':	Initialize();		break;

}

break;

case 'R':

case 'r':

printf("\rEnter Ram Routine to run:R");

M1 = _getkey();

switch(M1)

{

case 'R':

case 'r':	BurstRamRead();	break;

case 'W':

case 'w':	BurstRamWrite();	break;

}

break;

}

}

}