;******************************************************************************

;

;    Filename:		Listing1.asm (Absolute Code Version)

;    Date:    		10/10/03

;    File Version:  1.0

;

;    Author:        Ted Salazar

;    Company:       Maxim

;

;******************************************************************************

;

;	Program Description:

;						This program interfaces the internal I2C MSSP

;						(Peripheral) of the PIC18F442 to the MAX6955 I2C

;						compatible LED display driver. The program initializes

;						the MAX6955 driver for eight 14-segment LEDs. A

;						continuous scrolling message ,"THE QUICK BROWM FOX

;						JUMPS OVER THE LAZY DOG", is sent across the LED

;						segments to demonstrate a working example of scrolling.

;

;						To modify the message:

;						1)Put your message in the lookup table

;						2)Modify the value of CharCount for the message length

;

;******************************************************************************

;

; History:

; 09/15/03:	Improved code loop to follow the equation:

;			CharCount - 7 = ShiftCountPlusOne

; 08/24/03: Cleaned up code and commented.

; 08/03/03: Scrolling one MAX6955 now works.

; 07/23/04: Initialized MAX6955

;******************************************************************************

;

;    Files required:         P18F442.INC

;

;******************************************************************************

radix hex               ;Default to HEX

LIST P=18F442, F=INHX32	;Directive to define processor and file format

#include <P18F442.INC>	;Microchip's Include File

;******************************************************************************

;******************************************************************************

;

xmit    equ		06 ; Asynchronous TX is at C6

;******************************************************************************

;Configuration bit settings during Program Time

;

; Oscillator: 			EC-OSC2 as Clock Out (4MHz)

; Power Up Timer:		Enabled

; All other parameters:	Disabled

;******************************************************************************

;Variable definitions

; These variables are only needed if low priority interrupts are used.

; More variables may be needed to store other special function registers used

; in the interrupt routines.

CBLOCK	0x080

WREG_TEMP	;variable used for context saving

STATUS_TEMP	;variable used for context saving

BSR_TEMP	;variable used for context saving

;

ENDC

CBLOCK	0x000

EXAMPLE		;example of a variable in access RAM

;

data0

xmtreg

cntrb

cntra

bitctr

tcount

xcount

ycount

zcount

TableCount

CharCount

ShiftCountPlusOne

;

ENDC

;******************************************************************************

;Reset vector

; This code will start executing when a reset occurs.

ORG	0x0000

goto	Main		;go to start of main code

;******************************************************************************

;High priority interrupt vector

; This code will start executing when a high priority interrupt occurs or

; when any interrupt occurs if interrupt priorities are not enabled.

ORG	0x0008

bra	HighInt		;go to high priority interrupt routine

;******************************************************************************

;Low priority interrupt vector and routine

; This code will start executing when a low priority interrupt occurs.

; This code can be removed if low priority interrupts are not used.

ORG	0x0018

movff	STATUS,STATUS_TEMP	;save STATUS register

movff	WREG,WREG_TEMP		;save working register

movff	BSR,BSR_TEMP		;save BSR register

;	*** low priority interrupt code goes here ***

movff	BSR_TEMP,BSR		;restore BSR register

movff	WREG_TEMP,WREG		;restore working register

movff	STATUS_TEMP,STATUS	;restore STATUS register

retfie

;******************************************************************************

;High priority interrupt routine

; The high priority interrupt code is placed here to avoid conflicting with

; the low priority interrupt vector.

HighInt:

;	*** high priority interrupt code goes here ***

retfie	FAST

;******************************************************************************

;Start of main program

Main:

;	*** main code goes here ***

start

movlw   0x0FF

movwf   PORTB

clrf    PORTA

movlw   0x06        ;T Configure PortA as Digital

movwf   ADCON1

movlw   0x00FB 		;T A2 OUTPUT, ALL OTHERS INPUT

movwf   TRISA

movlw   0x0001    	;T B0 INPUT, ALL OTHERS OUTPUT

movwf   TRISB

movlw   0x00BF      ;T C6 (TX) OUTPUT, ALL OTHER C's INPUTs

movwf   TRISC

;******************************************************************************

HWI2C_init			;T I2C MSSP Initialization for MAX6955 App Note schematic

movlw   0x0028		;T turn MSSP ON & I2C MASTER

movwf   SSPCON1 	;T used to be sspcon

movlw   0x0000      ;T I2C FAST Mode Slew Rate Setting

movwf   SSPSTAT 	;T used to be sspstat

movlw   0x00FF      ;T SSPADD = (FOSC/(4*SCL)) -1 => 3900Hz @ 4MHz

movwf   SSPADD  	;T used to be sspadd

;******************************************************************************

;

MAX6955_Initialization

;Initialize 6955 Digits 0 to 7 as 14-Segments (Register 0x0C)

call   	HWI2C_start				;T I2C start command

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

movlw   0xCA         			;T load w with slave address

call    HWI2C_W_write			;T I2C write command

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

movlw   0x0C         			;T load w with reg address

call    HWI2C_W_write			;T I2C write command

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

movlw   0xFF         			;T load w with literal

call    HWI2C_W_write			;T I2C write command

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

call 	HWI2C_stop				;T I2C stop command

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

;Initialize 6955 Scan-Limit for Digits 0 to 7 (Register 0x03)

call   	HWI2C_start				;T I2C start command

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

movlw   0xCA         			;T load w with slave address

call    HWI2C_W_write			;T I2C write command

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

movlw   0x03         			;T load w with reg address

call    HWI2C_W_write			;T I2C write command

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

movlw   0x07         			;T load w with literal

call    HWI2C_W_write			;T I2C write command

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

call 	HWI2C_stop				;T I2C stop command

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

;Take 6955 out of shutdown (Register 0x04)

call   	HWI2C_start				;T I2C start command

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

movlw   0xCA         			;T load w with slave address

call    HWI2C_W_write			;T I2C write command

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

movlw   0x04         			;T load w with reg address

call    HWI2C_W_write			;T I2C write command

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

movlw   0x01         			;T load w with literal

call    HWI2C_W_write			;T I2C write commmand

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

call 	HWI2C_stop				;T I2C stop command

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

MAX6955_Scroll_Message

resetloop

movlw	0x00		;T 0x00 to w

movwf	TableCount	;T 0x00 to TableCount

;T**********************************************************

;T** Modify CharCount below for your message length*********

;T Scroll message...

;T "       THE QUICK BROWM FOX JUMPS OVER THE LAZY DOG       ",

;T 57 chars in decimal including spaces

movlw	d'57'

movwf	CharCount

;T**********************************************************

;T**********************************************************

;T Subtract 7 from CharCount to calculate ShiftCountPlusOne

movlw	0x07  					;T load W with 7

subwf 	CharCount,w				;T subtract 7 from CharCount

;										;T result in W

movwf	ShiftCountPlusOne		;T move W to ShiftCountPlusOne

shiftloop								;T Shift chars to scroll

call   	HWI2C_start				;T I2C Start

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

movlw   0xCA         			;T load w with slave address

call    HWI2C_W_write			;T I2C write command

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

movlw   0x60         			;T load w with reg address

call    HWI2C_W_write			;T I2C write command

btfsc   STATUS,C		 		;T Is there an error?

goto    errsrv

incf 	TableCount,1 			;T increment TableCount

movf	TableCount,0 			;T new TableCount to w

bsf		PCLATH,3		;T select  0x08XX address space

call    table			;T Call look up table for scroll data

;T Entry: TableCount in w

;T Exit: data in w

bcf		PCLATH,3		;T select current address space on return

call    HWI2C_W_write	;T digit0 write

btfsc   STATUS,C		;T Is there an error?

goto    errsrv

incf 	TableCount,1	;T increment TableCount

;								;T result in TableCount

incf 	TableCount,1	;T increment TableCount

;								;T result in TableCount

movf	TableCount,0	;T TableCount to w

bsf		PCLATH,3		;T select  0x08XX address space

call    table			;T Get data to scroll

;T Entry: TableCount in w

;T Exit: data in w

bcf		PCLATH,3		;T select current address space on return

call    HWI2C_W_write	;T digit1 write

btfsc   STATUS,C		;T Is there an error?

goto    errsrv

incf 	TableCount,1	;T increment TableCount

;								;T result in TableCount

incf 	TableCount,1	;T increment TableCount

;								;T result in TableCount

movf	TableCount,0	;T TableCount to w

bsf		PCLATH,3		;T select  0x08XX address space

call    table			;T Get data to scroll

;T Entry: TableCount in w

;T Exit: data in w

bcf		PCLATH,3		;T select current address space on return

call    HWI2C_W_write	;T digit 2 write

btfsc   STATUS,C		;T Is there an error?

goto    errsrv

incf 	TableCount,1	;T increment TableCount

;								;T result in TableCount

incf 	TableCount,1	;T increment Tablecount

;								;T result in TableCount

movf	TableCount,0	;T TableCount to w

bsf		PCLATH,3		;T select  0x08XX address space

call    table			;T Get data to scroll

;T Entry: TableCount in w

;T Exit: data in w

bcf		PCLATH,3		;T select current address space on return

call    HWI2C_W_write	;T digit 3 write

btfsc   STATUS,C		;T Is there an error?

goto    errsrv

incf 	TableCount,1	;T increment TableCount

;								;T result in TableCount

incf 	TableCount,1	;T increment TableCount

;								;T result in TableCount

movf	TableCount,0	;T TableCount to w

bsf		PCLATH,3		;T select  0x08XX address space

call    table			;T Get data to scroll

;T Entry: TableCount in w

;T Exit: data in w

bcf		PCLATH,3		;T select current address space on return

call    HWI2C_W_write	;T digit 4 write

btfsc   STATUS,C		;T Is there an error?

goto    errsrv

incf 	TableCount,1	;T increment TableCount

;								;T result in TableCount

incf 	TableCount,1	;T increment TableCount

;								;T result in TableCount

movf	TableCount,0	;T TableCount to w

bsf		PCLATH,3		;T select  0x08XX address space

call    table			;T Get data to scroll

;T Entry: TableCount in w

;T Exit: data in w

bcf		PCLATH,3		;T select current address space on return

call    HWI2C_W_write	;T digit 5 write

btfsc   STATUS,C		;T Is there an error?

goto    errsrv

incf 	TableCount,1	;T increment TableCount

;								;T result in TableCount

incf 	TableCount,1	;T increment TableCount

;								;T result in TableCount

movf	TableCount,0	;T TableCount to w

bsf		PCLATH,3		;T select  0x08XX address space

call    table			;T Get data to scroll

;T Entry: TableCount in w

;T Exit: data in w

bcf		PCLATH,3		;T select current address space on return

call    HWI2C_W_write	;T digit 6 write

btfsc   STATUS,C		;T Is there an error?

goto    errsrv

incf 	TableCount,1	;T increment TableCount

;								;T result in TableCount

incf 	TableCount,1	;T increment TableCount

;								;T result in TableCount

movf	TableCount,0	;T TableCount to w

bsf		PCLATH,3		;T select  0x08XX address space

call    table			;T Get data to scroll

;T Entry: TableCount in w

;T Exit: data in w

bcf		PCLATH,3		;T select current address space on return

write7	call    HWI2C_W_write	;T digit 7 write

btfsc   STATUS,C		;T Is there an error?

goto    errsrv

incf 	TableCount,1	;T increment TableCount

call 	HWI2C_stop		;

btfsc   STATUS,C		;T Is there an error?

goto    errsrv

call	delay

;T Subtract 14 from TableCount to reinitialize Tablecount

movlw	0x0E  			;T move decimal 14 to W

subwf 	TableCount,1	;T subtract 14 from TableCount

;								;T result in TableCount

decf	ShiftCountPlusOne,1	;T decrement ShiftCountPlusOne

;T result in ShiftCountPlusOne

; Check to see if ShiftCountPlusOne is zero yet?

movlw   0x00  					;T move 0 to W

subwf	ShiftCountPlusOne,0 	;T subtract 0 from ShiftCountPlusOne

;T to see if ShiftCountPlusOne is zero

;T result in W

btfss	STATUS,Z		;T Check Zero Flag to see if ShiftCountPlusOne is zero

;T skip if set meaning skip if ShiftCountPlusOne is zero

;T execute the next line if ShiftCountPlusOne is not zero

goto    shiftloop		;T Message not done yet continue shifting chars

goto    resetloop       ;T reset loop and loop forever

;******************************************************************************

errsrv

movlw   0x65			;T load w with 'e' = 0x65

movwf   xmtreg          ;T move w to xmtreg

call    asyxmtc			;T call UART routine

dead    goto    dead           	;T goto dead

;******************************************************************************

HWI2C_start

movlw	0x00				;T load w with literal

movwf	tcount				;T move w to tcount

bcf     PIR1,SSPIF 			;T clear completed flag

bsf     SSPCON2,SEN 		;T send start

sloop1

;       call 	delay_100ms			;OP_T

btfsc   PIR1,SSPIF 			;T start completed?

goto    sxloop          	;T exit loop

incf 	tcount,f			;T increment tcount

movlw	0xFF				;T load w with literal

subwf	tcount,w			;T subtract 0xFF from tcount result in w

btfss	STATUS,Z			;T test zero flag, skip next instr if z set

goto    sloop1        		;T loop until timeout

goto 	serror				;T complete timeout error

sxloop  call 	clear_cf_ok 		;T clear carry flag =>Success

return

serror  call 	set_cf_error 		;T set carry flag =>error

return

;******************************************************************************

HWI2C_stop

movlw	0x00				;T load w with literal

movwf	tcount				;T move w to tcount

bcf     PIR1,SSPIF 			;T clear completed flag

bsf     SSPCON2,PEN 		;T send stop

ploop1

;       call 	delay_100ms			;OP_T

btfsc   PIR1,SSPIF 			;T start completed?

goto    pxloop          	;T exit loop

incf 	tcount,f			;T increment tcount

movlw	0xFF				;T load w with literal

subwf	tcount,w			;T subtract 0xFF from tcount result in w

btfss	STATUS,Z			;T test zero flag, skip next instr if z set

goto    ploop1        		;T loop until timeout

goto 	perror				;T complete timeout error

pxloop  call 	clear_cf_ok 		;T clear carry flag =>Success

return

perror  call 	set_cf_error 		;T set carry flag =>error

return

;******************************************************************************

HWI2C_repeatedStart

movlw	0x00				;T load w with literal

movwf	tcount				;T move w to tcount

bcf     PIR1,SSPIF 			;T clear completed flag

bsf     SSPCON2,RSEN 		;T send repeated start

rsloop1

;       call 	delay_100ms			;OP_T

btfsc   PIR1,SSPIF 			;T repeated start completed?

goto    rsxloop          	;T exit loop

incf 	tcount,f			;T increment tcount

movlw	0xFF				;T load w with literal

subwf	tcount,w			;T subtract 0xFF from tcount result in w

btfss	STATUS,Z			;T test zero flag, skip next instr if z set

goto    rsloop1        		;T loop until timeout

goto 	rserror				;T complete timeout error

rsxloop call 	clear_cf_ok 		;T clear carry flag =>Success

return

rserror call 	set_cf_error 		;T set carry flag =>error

return

;******************************************************************************;

HWI2C_W_write

bcf     PIR1,SSPIF 			;T clear completed flag

movwf   SSPBUF     			;T send data

wloop1  btfss   PIR1,SSPIF 			;T data sent?

goto    wloop1				;T loop until completed

btfsc 	SSPCON2,ACKSTAT		;T ACK error? if no then skip

goto	werror			    ;T set_cf_error

call 	clear_cf_ok 		;T clear carry flag =>Success

return

werror  call 	set_cf_error 		;T set carry flag =>error

return

;******************************************************************************

set_cf_error

movlw 	0x00	 ;T 0x00 into W

sublw 	0x00     ;T Subtract W-0x00: If W<=N C set; If W>N C clear.

return           ;T error=> cf=set

;******************************************************************************

clear_cf_ok

movlw 	0x01	 ;T 0x01 into W

sublw 	0x00     ;T Subtract W-0x00: If W<=N C set; If W>N C clear.

return           ;T success=> cf=clear

;******************************************************************************

HWI2C_readMore_W

movlw	0x00				;T load w with literal

movwf	tcount				;T move w to tcount

bcf     PIR1,SSPIF 			;T clear completed flag

bsf     SSPCON2,RCEN 		;T receive data byte

rloop1

;       call 	delay_100ms			;OP_T

btfsc   PIR1,SSPIF 			;T data byte received?

goto    xloop          		;T exit loop

incf 	tcount,f			;T increment tcount

movlw	0xFF				;T load w with literal

subwf	tcount,w			;T subtract 0xFF from tcount result in w

btfss	STATUS,Z			;T test zero flag, skip next instr if z set

goto    rloop1        		;T loop until timeout

goto 	rerror				;T receive timeout error

xloop   movf    SSPBUF,w   			;T load data byte into w

movwf   data0				;T loads data byte into data0

bcf     PIR1,SSPIF 			;T clear completed flag

bcf     SSPCON2,ACKDT 		;T set ACK

bsf     SSPCON2,ACKEN 		;T send ACK

rloop2  btfss   PIR1,SSPIF 			;T send completed?

goto    rloop2              ;T loop until completed

call 	clear_cf_ok 		;T clear carry flag =>Success

movf	data0,w				;T move contents of data0 to w

return

rerror  call 	set_cf_error 		;T set carry flag =>error

return

;******************************************************************************

HWI2C_readLast_W

movlw	0x00				;T load w with literal

movwf	tcount				;T move w to tcount

bcf     PIR1,SSPIF 			;T clear completed flag

bsf     SSPCON2,RCEN 		;T receive data byte

rloop3

;       call 	delay_100ms			;OP_T

btfsc   PIR1,SSPIF 			;T data byte received?

goto    xloop2         		;T exit loop

incf 	tcount,f			;T increment tcount

movlw	0xFF				;T load w with literal

subwf	tcount,w			;T subtract 0xFF from tcount result in w

btfss	STATUS,Z			;T test zero flag, skip next instr if z set

goto    rloop3        		;T loop until timeout

goto 	rerror2				;T receive timeout error

xloop2  movf    SSPBUF,w   			;T load data byte into w

movwf   data0				;T loads data byte into data0

bcf     PIR1,SSPIF 			;T clear completed flag

bsf     SSPCON2,ACKDT 		;T set NOACK

bsf     SSPCON2,ACKEN 		;T send NOACK

rloop4  btfss   PIR1,SSPIF 			;T send completed?

goto    rloop4              ;T loop until completed

call 	clear_cf_ok 		;T clear carry flag =>Success

movf	data0,w				;T move contents of data0 to w

return

rerror2 call 	set_cf_error 		;T set carry flag =>error

return

;******************************************************************************

; UART routine

asyxmtc bcf     PORTC,xmit  ;T used to be portc,xmit

call    full

movlw   0x08        ;TEST_T "08"

movwf   bitctr

asyxmt1 rrcf    xmtreg,f

btfsc   STATUS,C

goto    asyxmt2

bcf     PORTC,xmit ;T used to be portc,xmit

goto    asyxmt3

asyxmt2 bsf     PORTC,xmit ;T used to be portc,xmit

;

asyxmt3 call    full

decfsz  bitctr,f

goto    asyxmt1

;

bsf     PORTC,xmit ;T used to be portc,xmit

call    full

retlw   0

;******************************************************************************

; 2400 Baud Rate @ 4MHz system clock

;

full    movlw   d'3'

movwf   cntrb

vdly0   movlw   d'43'

movwf   cntra

vdly1   decfsz  cntra,f

goto    vdly1

decfsz  cntrb,f

goto    vdly0

retlw   0

;******************************************************************************

;Delay subroutine to control the speed the message is shifted.

;The delays are based on using a 4MHz System Clock.

;

; Fast Reading: 	zcount = 0x02

;					ycount = 0x50

;					xcount = 0xFF

; Medium Reading:

;					zcount = 0x02

;					ycount = 0x7F

;					xcount = 0xFF

; Slow Reading:

;					zcount = 0x02

;					ycount = 0xF0

;					xcount = 0xFF

;

delay

movlw 	0x02		;T set W for zcount

movwf	zcount		;T to zcount

yagain	movlw 	0x7F		;T set W for ycount

movwf	ycount		;T to ycount

xagain	movlw 	0xFF		;T 0xFF to w

movwf	xcount		;T to xcount

repeat  decfsz	xcount,f	;T decrement xcount and skip if '0'

goto	repeat		;T not '0' yet

decfsz  ycount,f	;T decrement ycount and skip if '0'

goto	xagain		;T loop to xagain

decfsz  zcount,f	;T decrement zcount and skip if '0'

goto	yagain		;T loop to yagain

return

;******************************************************************************

; lookup table subroutine

; Update this lookup table with your message!

; On Entry: TableCount in W

; On Exit: W contains next data to scroll

ORG	0x0800

;*****

table

;*****

addwf	PCL,1	;add w to pc where 1 = file register

retlw	0x20	;"blank space"

retlw	0x20	;"blank space"

retlw	0x20	;"blank space"

retlw	0x20	;"blank space"

retlw	0x20	;"blank space"

retlw	0x20	;"blank space"

retlw	0x20	;"blank space"

retlw	0x54	;T

retlw	0x48	;H

retlw	0x45	;E

retlw	0x20	;"blank space"

retlw	0x51	;Q

retlw	0x55	;U

retlw	0x49	;I

retlw	0x43	;C

retlw	0x4B	;K

retlw	0x20	;"blank space"

retlw	0x42	;B

retlw   0x52	;R

retlw	0x4f	;O

retlw 	0x57	;W

retlw	0x4e	;N

retlw	0x20	;"blank space"

retlw 	0x46	;F

retlw	0x4f	;O

retlw 	0x58	;X

retlw	0x20	;"blank space"

retlw	0x4A	;J

retlw	0x55	;U

retlw	0x4D	;M

retlw	0x50	;P

retlw	0x53	;S

retlw	0x20	;"blank space"

retlw	0x4f	;O

retlw	0x56	;V

retlw	0x45	;E

retlw   0x52	;R

retlw	0x20	;"blank space"

retlw	0x54	;T

retlw	0x48	;H

retlw	0x45	;E

retlw	0x20	;"blank space"

retlw	0x4C	;L

retlw	0x41	;A

retlw	0x5A	;Z

retlw	0x59	;Y

retlw	0x20	;"blank space"

retlw	0x44	;D

retlw	0x4f	;O

retlw	0x47	;G

retlw	0x20	;"blank space"

retlw	0x20	;"blank space"

retlw	0x20	;"blank space"

retlw	0x20	;"blank space"

retlw	0x20	;"blank space"

retlw	0x20	;"blank space"

retlw	0x20	;"blank space"

;******************************************************************************

;End of program

END